## Network Segmentation and Scalability Analysis

### Introduction

As the network administrator for the Department of Motor Vehicles, I was tasked with two key network modifications: implementing a free guest wireless internet for waiting customers and installing security cameras to monitor the front desk and waiting area. These additions required careful network segmentation to ensure that guest and video traffic remained isolated from the existing internal data network. This report documents the reconfiguration process, explains how network traffic was segmented, and discusses scalability considerations for the guest wireless network.

### How Network Traffic Was Segmented

To meet the project requirements, I used Virtual Local Area Networks (VLANs) to segment traffic into separate broadcast domains. VLANs allow multiple logical networks to coexist on the same physical switch infrastructure while preventing traffic from crossing between them unless explicitly routed.

Existing VLANs

The network already had two VLANs configured:

- VLAN 50 (Data): Used for internal office data traffic on ports Fa0/5 and Fa0/6

- VLAN 150 (Voice): Reserved for future Voice over IP (VoIP) deployment on port Fa0/15

New VLANs Created

I added two new VLANs to support the project requirements:

- VLAN 70 (Guest): Dedicated to guest wireless traffic

- VLAN 80 (Video): Dedicated to security camera traffic

Port Assignment Strategy

The switch ports were assigned as follows:

- Port Fa0/7: Connected to the WRT300N wireless router and assigned to VLAN 70 (Guest). All guest devices connecting through this router are automatically placed on the Guest VLAN.

- Ports Fa0/8 and Fa0/9: Connected to the two webcams (Lobby and Door) and assigned to VLAN 80 (Video). Each camera received a static IP address in the 192.168.80.0/24 subnet.

- Ports Fa0/5 and Fa0/6: Remained on VLAN 50 (Data) for the existing desktop PCs.

Verification of Segmentation

To confirm that segmentation was working properly, I conducted ping tests from PC1 (located on VLAN 50 - Data) to devices on other VLANs:

- PC1 to Webcam Door (192.168.80.10): All four ping requests timed out, confirming that Data VLAN traffic cannot reach the Video VLAN.

- PC1 to Smartphone (192.168.70.102): All four ping requests timed out, confirming that Data VLAN traffic cannot reach the Guest VLAN.

These failed pings demonstrate successful Layer 2 isolation between VLANs. Without a router configured to route between these VLANs (which was not part of this project's requirements), traffic cannot cross from one VLAN to another. This ensures that guest users cannot access internal data network resources, and security camera footage remains isolated from both guest and data traffic.

Successful Guest Network Functionality

To confirm that the guest network itself was functioning correctly, I performed additional ping tests from within the Guest VLAN:

- Smartphone to Tablet (192.168.70.102 to 192.168.70.100): Successful replies confirmed that devices on the same VLAN can communicate.

- Smartphone to Gateway (192.168.70.1): Successful replies confirmed that guest devices can reach the wireless router for DHCP and internet access.

### Scalability of the Guest Wireless Network

The guest wireless network was designed with scalability in mind to accommodate the typical DMV environment, where customer volume fluctuates throughout the day and wait times average 30 to 60 minutes.

IP Addressing Scheme

I selected the 192.168.70.0/24 subnet for the guest network. This Class C private address range provides up to 254 usable host addresses. However, based on the scenario requirements, I limited the DHCP pool to a maximum of 70 concurrent users. This conservative limit is appropriate for a DMV office where physical seating capacity typically ranges from 30 to 50 customers. The 70-user cap provides a comfortable buffer without wasting IP address space.

DHCP Scope Configuration

The DHCP server on the WRT300N wireless router was configured with the following parameters:

- Start IP Address: 192.168.70.100

- Maximum Users: 70

- Client Lease Time: 240 minutes (4 hours)

Why Start at 192.168.70.100?

Starting the DHCP pool at .100 is a deliberate design choice. The addresses from .1 to .99 are reserved for static assignments:

- 192.168.70.1: Assigned to the WRT300N router as the default gateway

- 192.168.70.2 - 192.168.70.99: Reserved for potential future static devices such as additional access points, network printers, or management interfaces

This reservation strategy prevents IP address conflicts and allows for network growth without reconfiguring the DHCP scope.

Conclusion

The network reconfiguration successfully met all project requirements. VLAN segmentation was implemented on the existing switch, creating isolated broadcast domains for guest wireless traffic (VLAN 70) and video surveillance traffic (VLAN 80). The guest wireless network was configured on the repurposed WRT300N router with an open, unencrypted SSID named "DMV_Guest," DHCP starting at 192.168.70.100, a 70-user limit, and a 4-hour lease time optimized for DMV customer traffic patterns. Ping tests confirmed both successful isolation between VLANs and proper functionality within the guest network. This design is scalable, secure, and ready for deployment.